REAL TIME WIRE-FRAME POTHOLE DETECTION SYSTEM

PROJECT REPORT – PHASE I

Submitted by

MOHAMED B SIRAJUDDEEN					Register No.: 22TD0053

Under the guidance of

Dr. R.G SURESH KUMAR

in partial fulfillment of the requirements for the degree

of

BACHELOR OF TECHNOLOGY

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

RAJIV GANDHI COLLEGE OF ENGINEERING AND TECHNOLOGY

(Approved by AICTE and Affiliated to Pondicherry University)

Accredited with ’A+’ Grade in Cycle II by NAAC

PUDUCHERRY-607402

NOVEMBER 2025

# RAJIV GANDHI COLLEGE OF ENGINEERING AND TECHNOLOGY

(Approved by AICTE and Affiliated to Pondicherry University)

Accredited with ’A+’ Grade in Cycle II by NAAC

DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

## BONAFIDE CERTIFICATE

This is to certify that the project work entitled “REAL TIME WIRE-FRAME POTHOLE DETECTION SYSTEM” is a bonafide work done by MOHAMED B SIRAJUDDEEN [REGISTER No.:22TD0053] in partial fulfillment of the requirement, for the award of B.Tech Degree in Computer Science and Engineering by Pondicherry University during the academic year 2025-2026.

## PROJECT GUIDE	HEAD OF THE DEPARTMENT

ACKNOWLEDGEMENT

We are very thankful and grateful to our beloved guide, Dr. R.G SURESH KUMAR whose great support in valuable advices, suggestions and tremendous help enabled us in completing our project. He has been a great source of inspiration to us.

We also sincerely thank our Head of the Department, Dr. R.G SURESH KUMAR whose continuous encouragement and sufficient comments enabled us to complete our project report.

We thank all our Staff members who have been by our side always and helped us with our project.  We also sincerely thank all the lab technicians for their help as in the course of our project development.

We would also like to extend our sincere gratitude and grateful thanks to our Principal, 
Dr. E. VIJAYAKRISHNA RAPAKA for having extended the Research and Development facilities of the department.

We are grateful to our Chairman, Shri. M.K. RAJAGOPALAN. He has been a constant source of inspiration right from the beginning.

We wish to thank our family members and friends for their constant encouragement, constructive criticisms and suggestions that has helped us in timely completion of this project.

ii

ABSTRACT

Road surface irregularities like potholes present major risks to vehicle safety, passenger comfort, and the efficiency of traffic flow. These issues have significant implications in the transportation domain, as they directly affect road safety, traffic management, and overall driving experience. With the growth of autonomous vehicles and smart city initiatives, the need for intelligent systems that can accurately detect and analyze road conditions in real time has become increasingly important.

Traditional approaches for pothole detection have been enhanced through the integration of computer vision and artificial intelligence. These systems utilize cameras to capture road images, which are then processed using deep learning models to detect and classify potholes. By applying AI-based image analysis, the existing methods effectively automate the identification of road surface irregularities, offering a faster and more efficient alternative to manual inspections and sensor-based techniques.

The limitation in existing systems is their reliance on two-dimensional bounding boxes or simple classification, which lack depth and shape information. To address this limitation, my project proposes a Real-Time Wireframe Pothole Detection System that generates a three-dimensional wireframe of road surfaces using camera input. The proposed model aims to integrate depth estimation with wireframe modeling for precise analysis of surface irregularities, enabling better decisions for autonomous vehicles and improving road safety.

Keywords: Real-time detection, Pothole detection, Wireframe modeling, Computer vision, Deep learning.

# iii

# LIST OF TABLES

| TABLE NO. | NAME OF THE TABLE | PAGE NO. |
| --- | --- | --- |
| 2.6 | Literature Survey Comparison | 15 |

iv

# LIST OF FIGURES

| FIGURE NO. | NAME OF THE FIGURE | PAGE NO. |
| --- | --- | --- |
| 1.1 1.2 1.3 1.4 3.1 6.1 7.1 7.2 | Deep Learning Architecture Machine Learning Vs Deep Learning Neural Network Architecture Unrolled RNN Architecture of Existing System Architecture of Proposed System Structure Comparison Graphs Overall Graphs Comparison | 3 4 5 8 17 24 35 35 |

v

# LIST OF ABBREVIATIONS

Al	-	Artificial Intelligence ML	-	Machine Learning

NLP	-	Natural Language Processing ANN	-	Artificial Neural Network CNN	-	Convolutional Neural Network RNN	-	Recurrent Neural Network

GAN	-	Generative Adversarial Network SVM	-	Support Vector Machine

RL	-	Reinforcement Learning DL	-	Deep Learning

OCR	-		Optical Character Recognition AGI	-	Artificial General Intelligence CV	-	Computer Vision

DNN	-	Deep Neural Network

vi

TABLE OF CONTENTS

| CHAPTER NO. | TITLE | PAGE NO. |
| --- | --- | --- |

ABSTRACT                                               				iii

LIST OF TABLES						        	iv

LIST OF FIGURES						        	v

LIST OF ABBREVIATIONS		vi

- INTRODUCTION							1

  - ARTIFICIAL INTELLIGENCE				1

  - AUTONOMOUS VEHICLES				2

    - Road Surface Perception

    - Vision-Based Structural Understanding of 
 	Road Defects						8

    - Real-Time Structural Modeling			9

  - COMPUTER VISION SYSTEM ARCHITECTURE	9

    - Image Acquisition and Preprocessing			9

    - Deep Learning-Based Pothole Detection		11

    - Monocular Depth Estimation				15

    - Wireframe and Geometric Modeling			17

  - REAL-TIME PROCESSING FRAMEWORK		17

    - Detection Pipeline and Data Flow			18

    - Temporal Consistency and Frame Stabilization	19

    - Computational Optimization for 
Real-Time Execution					20

  - SYSTEM APPLICATIONS AND USE CASES		20

    - Autonomous Navigation Assistance			20

    - Road Infrastructure Monitoring			20

    - Safety and Decision Support Systems		20

- LITERATURE SURVEY						20

  - LITERATURE SURVEY – I					22

  - LITERATURE SURVEY – II 				23

  - LITERATURE SURVEY – III 				23

  - LITERATURE SURVEY – IV 				23

  - LITERATURE SURVEY – V 				23

  - LITERATURE SURVEY COMPARISON			24

- SYSTEM STUDY							27

  - EXISTING SYSTEM						27

    - Issues in the system					28

    - Existing system architecture				29

    - Solutions						30

  - PROPOSED SYSTEM					31

- SYSTEM REQUIREMENTS					31

  - HARDWARE REQUIREMENTS				31

  - SOFTWARE REQUIREMENTS				32

  - ENVIRONMENT SETUP					33

5. 	CONCLUSION

7.1 	CONCLUSION						37

7.2 	FUTURE ENHANCEMENT					38

APPENDIX								39

REFERENCES							46

NOTE:

Font: Time New Roman (Chapter Heading Size 14 & Heading Size 12,Sub Heading Size 12)

- Page numbers are only for your reference- It can be increased based on your content.

- Table of contents- Sub chapters can be extended based on your project.

- References inside the project report should be mentioned, References should only cover the papers u referred. Avoid the website links.

- Diagrams should not copy from net, u have to draw and clearly mention the diagram numbers.

CHAPTER I

INTRODUCTION

  - ARTIFICIAL INTELLIGENCE

Artificial Intelligence (AI) is a branch of computer science that focuses on designing systems capable of performing tasks that normally require human intelligence. These tasks include perception, learning, reasoning, decision-making, and problem solving. AI systems are developed to analyze large volumes of data, identify meaningful patterns, and make predictions or decisions with minimal human intervention. With the rapid growth of computational power and availability of data, AI has evolved from rule-based systems to advanced learning-based models that can adapt and improve over time.

Modern AI heavily relies on techniques such as Machine Learning and Deep Learning, where algorithms learn directly from data instead of being explicitly programmed. These approaches enable systems to handle complex, unstructured data such as images, videos, audio, and sensor signals. As a result, AI has found widespread applications in domains such as healthcare, autonomous vehicles, robotics, finance, smart cities, and intelligent transportation systems.

In real-world scenarios, AI plays a crucial role in automating perception-driven tasks that require speed, accuracy, and consistency beyond human capability. In the context of road safety and autonomous navigation, AI enables vehicles to understand their surroundings, detect anomalies, and make informed decisions in real time. The integration of AI with computer vision allows machines to interpret visual information from cameras and extract meaningful insights, making it a foundational technology for advanced driver-assistance systems and intelligent infrastructure monitoring.

  - AUTONOMOUS VEHICLES

Autonomous vehicles are intelligent transportation systems capable of sensing their environment, interpreting surrounding conditions, and navigating without direct human control. These vehicles rely on a combination of sensors, embedded computing, artificial intelligence, and control algorithms to perceive the road, detect obstacles, and make real-time driving decisions. By continuously analyzing data from cameras and other sensing devices, autonomous vehicles aim to improve road safety, reduce human error, and enhance driving efficiency. They play a vital role in modern transportation by enabling features such as lane keeping, obstacle avoidance, adaptive speed control, and automated path planning. As autonomous driving technology advances, accurate perception of road conditions—including surface anomalies such as potholes—becomes critical for ensuring vehicle stability, passenger comfort, and safe navigation.

    - Road Surface Perception

Road surface perception is a fundamental and challenging component of autonomous vehicle research, as the physical condition of the road directly influences vehicle dynamics, safety, comfort, and decision-making. Unlike static obstacles such as vehicles, pedestrians, or traffic signs, road surface anomalies are part of the driving terrain itself and continuously interact with the vehicle’s wheels, suspension system, and control mechanisms. Surface irregularities such as potholes, cracks, depressions, bumps, and uneven pavement can significantly affect steering stability, braking efficiency, and ride comfort. Therefore, reliable perception of road surface conditions is essential for achieving safe and robust autonomous navigation.

In autonomous driving systems, perception modules are responsible for transforming raw sensor data into meaningful environmental understanding. Road surface perception extends beyond detecting the presence of obstacles; it involves analyzing subtle geometric and textural variations on the road that may indicate structural damage or degradation. Potholes, in particular, often lack sharp boundaries and may visually resemble shadows, stains, or water patches, making them difficult to identify using conventional object detection techniques. This complexity necessitates continuous and detailed analysis of the road surface rather than event-based detection alone.

Traditional autonomous driving pipelines primarily focus on lane detection, vehicle tracking, and obstacle avoidance. However, recent research highlights that neglecting road surface quality can lead to unsafe navigation decisions, especially at higher speeds or under adverse weather and lighting conditions. A pothole that is not detected in time may result in sudden vehicle jolts, loss of traction, or damage to mechanical components. Consequently, road surface perception has emerged as a critical research area within intelligent transportation systems and autonomous vehicle development.

Vision-based perception systems have gained significant attention for road surface analysis due to their cost-effectiveness, scalability, and compatibility with existing autonomous vehicle platforms. Cameras provide rich visual information, capturing texture, color variations, and shading patterns that can indicate surface anomalies. Vision-based systems analyze these visual cues to differentiate intact road surfaces from damaged regions. However, relying solely on raw image appearance is insufficient, as environmental factors such as illumination changes, shadows, reflections, rainwater, and motion blur can significantly degrade detection accuracy.

To overcome these challenges, modern road surface perception approaches incorporate deep learning techniques that can learn robust representations from large-scale datasets. Convolutional Neural Networks (CNNs) and transformer-based architectures are capable of extracting hierarchical features that encode both local texture details and global contextual information. These features enable the system to distinguish potholes from visually similar patterns such as oil stains or road markings. Nevertheless, while deep learning improves detection accuracy, many existing systems still operate in a purely two-dimensional space, limiting their understanding of the actual physical structure of the road surface.

Depth perception plays a vital role in enhancing road surface analysis, as surface anomalies are inherently three-dimensional in nature. A pothole is not merely a dark region on the road; it represents a depression with measurable depth, volume, and boundary geometry. Depth information allows autonomous vehicles to assess the severity of surface damage and decide whether avoidance or controlled traversal is safer. However, traditional depth sensing solutions such as LiDAR and stereo cameras introduce high hardware costs, increased power consumption, and calibration complexity, making them less suitable for widespread deployment in low-cost or mass-market autonomous systems.

Recent advances in monocular depth estimation have opened new possibilities for vision-based road surface perception. These techniques estimate depth from a single RGB camera by learning geometric priors from large datasets. By combining monocular depth estimation with pothole detection models, autonomous systems can infer the relative depth and shape of road anomalies without requiring specialized hardware. This approach significantly enhances the perception pipeline while maintaining cost efficiency and real-time performance.

Accurate road surface perception also contributes to proactive decision-making in autonomous vehicles. Rather than reacting to hazards after encountering them, perception-driven systems can anticipate road irregularities in advance and adjust driving behavior accordingly. This includes reducing vehicle speed, modifying steering trajectories, or selecting alternative paths to minimize risk. Early hazard anticipation reduces mechanical stress on vehicle components such as suspension and tires and improves passenger comfort by preventing abrupt maneuvers.

Another important aspect of road surface perception is temporal consistency. Road anomalies persist over time and appear across consecutive frames in video streams. Effective perception systems must maintain stable detection results across frames to avoid false positives or flickering predictions. Temporal modeling techniques, such as frame-to-frame tracking and motion smoothing, help improve reliability by leveraging continuity in visual and geometric information. This temporal awareness is particularly important for real-time autonomous navigation, where decisions must be both fast and stable.

Furthermore, road surface perception plays a significant role in infrastructure monitoring and intelligent transportation systems beyond autonomous vehicles. Data collected from perception systems can be aggregated to create detailed maps of road conditions, enabling predictive maintenance and efficient resource allocation by municipal authorities. Automated road condition assessment reduces reliance on manual inspections, lowers operational costs, and improves overall road safety for all users.

Despite substantial progress, several challenges remain in road surface perception research. Variations in road materials, lighting conditions, weather effects, and camera viewpoints introduce significant complexity. Additionally, the lack of large-scale, well-annotated datasets containing diverse road surface anomalies limits model generalization. Addressing these challenges requires continued research into robust feature extraction, adaptive learning techniques, and efficient real-time processing pipelines.

In summary, road surface perception is a critical enabling technology for autonomous vehicles and intelligent transportation systems. It bridges the gap between visual understanding and physical interaction with the driving environment. By enabling accurate detection, depth estimation, and structural interpretation of road anomalies such as potholes, road surface perception enhances vehicle safety, improves navigation reliability, and supports proactive decision-making. This growing importance motivates the development of advanced vision-based solutions capable of understanding road geometry in real time, forming the foundation for the proposed work in this project.

    - Vision-Based Structural Understanding of Road Defects

Recent research in autonomous driving and intelligent transportation systems increasingly emphasizes the need for vision-based pothole detection methods that extend beyond traditional two-dimensional object detection. While early approaches focused primarily on identifying the presence of potholes using bounding boxes or pixel-level segmentation, such representations provide limited information about the actual physical characteristics of road surface anomalies. Potholes are inherently three-dimensional structures that vary in depth, shape, volume, and boundary geometry. Understanding these structural properties is essential for reliable decision-making in autonomous vehicles, as the severity of a pothole directly determines the appropriate vehicle response.

Conventional 2D object detection techniques treat potholes as visual entities similar to other objects in a scene. Although such methods are computationally efficient and suitable for real-time deployment, they fail to capture the geometric complexity of road defects. A bounding box around a pothole does not convey information about how deep the pothole is, how steep its edges are, or how much of the road surface is affected. As a result, an autonomous vehicle may underestimate or overestimate the hazard posed by a pothole, leading to unsafe navigation decisions or unnecessary evasive maneuvers.

To overcome these limitations, recent studies propose incorporating structural understanding into pothole detection systems. Structural understanding refers to the ability of a perception system to infer the three-dimensional shape and spatial configuration of a road defect rather than merely detecting its visual presence. This shift from appearance-based detection to geometry-aware perception marks a significant advancement in road surface analysis research.

Monocular camera-based systems have emerged as a promising solution for achieving structural understanding while maintaining cost efficiency. Unlike LiDAR and stereo vision systems, which require specialized hardware and complex calibration, monocular cameras are inexpensive, lightweight, and already widely deployed in vehicles. Advances in deep learning have made it possible to estimate depth information from a single RGB image by learning scene geometry from large datasets. These monocular depth estimation models infer relative depth by analyzing visual cues such as shading, texture gradients, perspective, and object scale.

By integrating monocular depth estimation with vision-based pothole detection, autonomous systems can obtain an approximate three-dimensional representation of road surface anomalies. This approach allows the system to estimate key structural attributes such as pothole depth, surface depression profile, and boundary contours. Although monocular depth estimation may not provide absolute metric accuracy comparable to LiDAR, it offers sufficient relative depth information for decision-making tasks such as speed regulation and trajectory planning.

The generation of wireframe or mesh representations further enhances the structural understanding of potholes. A wireframe model represents the geometric outline and depth variations of a pothole using a network of connected edges or vertices. This representation captures the shape and spatial extent of the defect more effectively than flat segmentation masks. Mesh-based representations can further approximate the surface topology by dividing the pothole region into interconnected polygons, enabling a more detailed geometric interpretation.

Wireframe modeling provides several advantages in autonomous navigation. First, it allows the perception system to differentiate between shallow surface irregularities and deep potholes that pose significant risks. Second, it enables precise localization of pothole boundaries, which is critical for planning safe vehicle trajectories around damaged regions. Third, wireframe representations support quantitative analysis, such as estimating the depth gradient and area affected by the pothole, which can be used to assess severity.

Structural insight obtained from wireframe-based modeling directly supports informed behavioral planning in autonomous vehicles. Instead of treating all detected potholes uniformly, the vehicle can adapt its response based on the estimated geometry of the road defect. For example, shallow potholes may be traversed safely at reduced speed, while deeper potholes may require lane adjustment or complete avoidance. This level of decision-making is not achievable with simple 2D detection methods.

Controlled deceleration is one of the primary behaviors enabled by structural pothole understanding. By estimating the depth and slope of a pothole, the vehicle can determine an appropriate speed reduction that minimizes suspension stress and passenger discomfort. Similarly, path adjustment decisions rely on accurate boundary estimation, ensuring that steering maneuvers avoid sharp pothole edges without causing abrupt lateral movements that could destabilize the vehicle.

Real-time processing is a critical requirement for structural pothole detection systems. Autonomous vehicles operate in dynamic environments where perception and decision-making must occur within milliseconds. Integrating object detection, depth estimation, and wireframe generation into a single real-time pipeline presents significant computational challenges. Efficient model design, lightweight neural architectures, and optimized inference strategies are essential to meet real-time constraints without sacrificing accuracy.

Another important aspect of real-time structural pothole detection is temporal consistency. Road anomalies persist across consecutive frames in a video stream, and structural estimates should remain stable over time. Temporal smoothing techniques and frame-to-frame consistency checks help reduce noise in depth estimation and prevent fluctuations in wireframe geometry. Stable structural representations improve decision reliability and prevent erratic vehicle behavior.

Beyond autonomous navigation, wireframe-based pothole modeling has broader applications in road infrastructure monitoring. Structural data collected by vehicles can be aggregated to create detailed three-dimensional maps of road conditions over large geographic areas. Such data can assist municipal authorities in prioritizing road maintenance, estimating repair costs, and monitoring infrastructure degradation over time. Vision-based structural analysis thus contributes not only to vehicle safety but also to smart city initiatives and intelligent transportation management.

Despite its advantages, vision-based structural pothole detection faces several challenges. Monocular depth estimation is sensitive to lighting variations, shadows, and surface textures, which can affect depth accuracy. Road surfaces exhibit significant variability in material, color, and reflectance, complicating feature extraction. Additionally, obtaining large-scale annotated datasets with ground-truth depth information for potholes remains difficult. Addressing these challenges requires robust model training, domain adaptation techniques, and careful system design.

In conclusion, the integration of structural understanding into vision-based pothole detection represents a crucial advancement in autonomous vehicle perception research. By moving beyond 2D object detection and incorporating depth estimation and wireframe modeling, autonomous systems gain a deeper understanding of road surface anomalies. This geometric insight enables safer, more informed behavioral planning, including controlled deceleration and path adjustment. As autonomous driving technology continues to evolve, real-time wireframe pothole detection is expected to play a vital role in enhancing navigation safety, vehicle durability, and overall transportation efficiency.

    - Real-Time Structural Modeling

Real-time structural modeling is an advanced perception capability that enables autonomous systems to interpret detected road anomalies in a geometric form suitable for navigation, control, and decision-making. Unlike traditional perception approaches that represent road defects as two-dimensional regions or bounding boxes, structural modeling captures the spatial characteristics of anomalies such as potholes, including depth, shape, slope, and boundary geometry. This geometric representation provides a more meaningful understanding of the physical interaction between the vehicle and the road surface, which is essential for safe and reliable autonomous vehicle operation.

In autonomous driving, perception and planning modules operate under strict real-time constraints. Structural modeling must therefore generate accurate geometric representations with minimal computational delay. The objective of real-time structural modeling is not to reconstruct the road surface with perfect physical accuracy, but to produce a sufficiently detailed approximation that supports immediate driving decisions. This balance between geometric fidelity and computational efficiency defines a key research challenge in real-time road surface analysis.

Wireframe and mesh-based representations are commonly employed techniques for structural modeling of road anomalies. A wireframe model represents the structure of a pothole using interconnected line segments that outline its boundaries and internal depth variations. This representation emphasizes edges, contours, and spatial relationships while remaining lightweight in terms of data complexity. Mesh-based models extend this concept by dividing the pothole surface into a set of connected polygons, enabling a more continuous approximation of surface topology. Both approaches provide a significant improvement over flat, two-dimensional representations by explicitly encoding three-dimensional geometry.

Structural modeling allows autonomous systems to assess the severity of potholes in a quantitative manner. Depth information reveals how far the road surface deviates from the nominal driving plane, while geometric boundaries define the affected area. These parameters enable the system to distinguish between minor surface imperfections and critical hazards that require immediate intervention. Without such modeling, autonomous vehicles may treat all detected potholes equally, leading to inefficient or unsafe responses.

One of the primary benefits of real-time structural modeling is its direct contribution to path planning and motion control. Autonomous vehicles rely on accurate environmental models to compute safe trajectories that avoid obstacles and minimize risk. When a pothole is represented as a geometric structure, the planning module can evaluate alternative paths that bypass the damaged region while maintaining smooth vehicle motion. The wireframe representation provides precise boundary information, enabling the vehicle to navigate around potholes with minimal lateral deviation.

Speed regulation is another critical application of structural modeling. The depth and slope of a pothole determine the level of mechanical stress imposed on the vehicle’s suspension and tires. By analyzing these geometric attributes, the autonomous system can compute an appropriate deceleration profile before reaching the anomaly. Controlled deceleration reduces the likelihood of sudden jolts and improves passenger comfort. In cases where avoidance is not possible, structural modeling helps determine whether a pothole can be traversed safely at reduced speed.

Obstacle avoidance in autonomous vehicles traditionally focuses on dynamic entities such as vehicles and pedestrians. However, road surface anomalies represent a unique class of obstacles that cannot be avoided using conventional object avoidance strategies alone. Structural modeling integrates road anomalies into the same planning framework as other obstacles, allowing the system to reason about them using geometric constraints. This unified representation improves the coherence and robustness of autonomous navigation.

Real-time performance is a fundamental requirement for structural modeling systems deployed in autonomous vehicles. The perception pipeline must process high-resolution visual data, perform detection, estimate depth, and generate structural representations within tight latency budgets. Achieving this requires optimized algorithms, efficient data structures, and hardware-aware implementations. Lightweight neural networks, parallel processing, and selective region modeling are commonly employed to meet real-time constraints.

Temporal consistency is another important aspect of real-time structural modeling. Road anomalies persist across consecutive frames, and their structural representation should remain stable over time. Inconsistent modeling can lead to oscillatory planning decisions or erratic vehicle behavior. Techniques such as temporal filtering, motion tracking, and frame-to-frame geometry alignment are used to maintain continuity in structural representations. Stable modeling improves confidence in the perception system and supports smoother control actions.

Structural modeling also enables higher-level reasoning about road conditions beyond immediate navigation. Aggregated structural data can be used to analyze long-term road degradation patterns and predict future maintenance needs. When vehicles continuously collect geometric representations of road anomalies, this information can be transmitted to centralized systems for large-scale infrastructure monitoring. Such applications extend the impact of real-time structural modeling from individual vehicle safety to broader smart city and transportation management initiatives.

Despite its advantages, real-time structural modeling faces several challenges. Variability in lighting conditions, surface materials, and weather can affect the reliability of depth estimation and geometric reconstruction. Computational limitations on embedded vehicle hardware constrain model complexity and resolution. Additionally, the lack of standardized evaluation metrics for structural road modeling complicates performance assessment. Ongoing research focuses on improving robustness, scalability, and accuracy under real-world conditions.

In summary, real-time structural modeling plays a crucial role in bridging the gap between perception and control in autonomous vehicles. By representing road anomalies such as potholes in a geometric form, autonomous systems gain a deeper understanding of the physical driving environment. Wireframe and mesh-based models enable accurate path planning, speed regulation, and obstacle avoidance, directly contributing to safer and more reliable vehicle operation. As autonomous driving technology advances, real-time geometric modeling of road surfaces is expected to become an essential component of next-generation perception systems and intelligent transportation solutions.

  - COMPUTER VISION SYSTEM ARCHITECTURE

Computer vision system architecture forms the backbone of intelligent perception in autonomous and driver-assistance systems. It defines how raw visual data acquired from sensors is transformed into meaningful representations that support decision-making. In the context of road surface analysis and pothole detection, the architecture must efficiently integrate image acquisition, feature extraction, depth estimation, and geometric modeling while maintaining real-time performance.

A well-designed computer vision architecture ensures modularity, scalability, and robustness. Each stage of the pipeline performs a specific function, allowing independent optimization and easier system upgrades. For real-time pothole detection, the architecture must balance computational efficiency with perceptual accuracy, as delayed or incorrect perception can lead to unsafe navigation decisions. The proposed system architecture focuses on a sequential yet tightly coupled pipeline that converts raw camera input into a structured wireframe representation of road surface anomalies.

    - Image Acquisition and Preprocessing

Image acquisition is the initial stage of the computer vision pipeline, where visual data of the road surface is captured using an RGB camera. In autonomous vehicle applications, cameras are typically mounted at fixed positions to provide a forward-facing view of the driving environment. The quality of captured images directly affects the performance of subsequent processing stages, making reliable acquisition essential.

Raw images often contain noise, illumination variations, motion blur, and perspective distortions caused by vehicle movement and environmental conditions. Preprocessing techniques are applied to enhance image quality and standardize input data for deep learning models. Common preprocessing steps include resizing, normalization, noise reduction, contrast enhancement, and color space conversion. These operations improve feature visibility and reduce sensitivity to lighting changes, shadows, and surface reflections.

Effective preprocessing ensures that the deep learning models focus on relevant road features rather than artifacts. In real-time systems, preprocessing must be computationally lightweight to avoid introducing latency. Therefore, only essential transformations are applied to maintain a balance between image enhancement and processing speed.

    - Deep Learning–Based Pothole Detection

Deep learning–based pothole detection is a core component of the computer vision architecture. Convolutional Neural Networks (CNNs) are widely used for this task due to their ability to automatically learn hierarchical features from visual data. These features capture texture patterns, edge information, and contextual cues that distinguish potholes from normal road surfaces.

Modern object detection models such as YOLO (You Only Look Once) are particularly suited for real-time applications. They perform detection in a single forward pass, enabling high-speed inference while maintaining acceptable accuracy. In pothole detection, the model identifies regions of interest where road surface anomalies are likely present.

Unlike traditional image processing methods that rely on handcrafted features, deep learning models adapt to diverse road conditions through training on large datasets. This adaptability improves robustness under varying lighting, weather, and pavement conditions. However, deep learning models primarily operate in two-dimensional space, necessitating additional processing to extract structural information from detected pothole regions.

    - Monocular Depth Estimation

Monocular depth estimation enhances the perception pipeline by providing three-dimensional context using a single RGB image. Depth estimation is critical for understanding the physical severity of potholes, as visual appearance alone cannot reliably indicate depth or volume.

Recent advances in deep learning have enabled accurate depth estimation from monocular images by learning geometric priors from large-scale datasets. These models analyze visual cues such as perspective, shading, and texture gradients to infer relative depth. Although monocular depth estimation does not provide absolute measurements comparable to LiDAR, it offers sufficient accuracy for navigation and decision-making.

Integrating depth estimation into the pothole detection pipeline allows the system to evaluate how deeply the road surface deviates from its normal plane. This information is essential for determining appropriate vehicle responses, such as speed reduction or path adjustment. Monocular depth estimation provides a cost-effective alternative to sensor-heavy systems, making it suitable for scalable autonomous applications.

    - Wireframe and Geometric Modeling

Wireframe and geometric modeling convert depth-enhanced pothole detections into structured representations of road surface anomalies. A wireframe model outlines the geometric boundaries and internal structure of a pothole using interconnected edges and vertices. This representation captures shape, size, and spatial extent while remaining computationally efficient.

Geometric modeling enables quantitative analysis of potholes, including boundary estimation and surface profiling. Unlike flat segmentation masks, wireframe representations preserve three-dimensional relationships, allowing the system to reason about how a pothole interacts with vehicle dynamics. This structured understanding is essential for downstream modules such as path planning and motion control.

In real-time systems, wireframe modeling must be optimized to avoid excessive computation. By focusing only on detected regions of interest, the system minimizes processing overhead while maintaining accurate structural representation.

  - REAL-TIME PROCESSING FRAMEWORK

The real-time processing framework governs how the computer vision pipeline operates under strict latency constraints. Autonomous vehicles require perception, analysis, and decision-making to occur within milliseconds to ensure safe operation. The framework coordinates data flow between modules and ensures timely execution of each processing stage.

Real-time performance is achieved through efficient model design, optimized data handling, and parallel execution where possible. The framework prioritizes critical perception tasks and minimizes redundant computation. This ensures that the system remains responsive even in complex driving environments.

    - Detection Pipeline and Data Flow

The detection pipeline defines the sequence of operations that transform raw camera input into actionable information. Visual data flows from image acquisition to preprocessing, pothole detection, depth estimation, and geometric modeling. Each stage produces intermediate outputs that serve as inputs to subsequent modules.

Efficient data flow management reduces memory overhead and latency. Intermediate representations are kept compact, and unnecessary data duplication is avoided. A well-structured pipeline ensures smooth integration of perception outputs with navigation and control systems.

    - Temporal Consistency and Frame Stabilization

Temporal consistency is essential for maintaining stable perception results across consecutive frames. Road anomalies persist over time, and their detection and structural representation should remain consistent. Without temporal stabilization, perception outputs may fluctuate, leading to erratic vehicle behavior.

Frame-to-frame consistency techniques such as temporal smoothing and region tracking reduce noise in detection and depth estimation. These techniques improve reliability and confidence in the perception system, enabling smoother control decisions.

    - Computational Optimization for Real-Time Execution

Computational optimization ensures that the perception system meets real-time constraints without sacrificing accuracy. Lightweight neural networks, reduced input resolutions, and selective processing of regions of interest are commonly employed optimization strategies.

Hardware acceleration using GPUs further enhances processing speed. Optimization techniques enable deployment on embedded platforms commonly used in autonomous vehicles, ensuring scalability and practical applicability.

  - SYSTEM APPLICATIONS AND USE CASES

The proposed computer vision system supports multiple applications within autonomous driving and intelligent transportation systems. Its ability to detect and structurally model road anomalies extends its usefulness beyond basic perception tasks.

    - Autonomous Navigation Assistance

In autonomous navigation, the system provides critical information about road surface conditions to support safe driving decisions. Structural modeling of potholes enables proactive speed adjustment and trajectory planning, reducing the risk of vehicle damage and passenger discomfort.

    - Road Infrastructure Monitoring

Aggregated perception data can be used for large-scale road condition monitoring. Structural representations of potholes collected by vehicles enable automated infrastructure assessment and predictive maintenance planning, reducing reliance on manual inspections.

    - Safety and Decision Support Systems

The system enhances safety by providing reliable, geometry-aware perception of road hazards. Decision support mechanisms use this information to trigger warnings, adjust driving behavior, or inform higher-level control systems. Accurate road surface perception directly contributes to safer and more resilient autonomous vehicle operation.

CHAPTER II

LITERATURE SURVEY

2.1 LITERATURE SURVEY – I

TITLE: An Enhanced YOLOv8 Model for Real-Time and Accurate Pothole Detection and Measurement

AUTHORS: Mustafa Yurdakul, Şakir Taşdemir

YEAR: 2025

DESCRIPTION: This paper proposes an enhanced YOLOv8-based segmentation model for real-time pothole detection and physical measurement using RGB-D data. The authors collect synchronized RGB and depth images using an Intel RealSense D415 camera and introduce the publicly available PothRGBD dataset labeled for segmentation. The standard YOLOv8n-seg model is improved by integrating Dynamic Snake Convolution (DSConv) for better handling of irregular pothole boundaries, the SimAM attention module to highlight informative regions, and the GELU activation function to improve learning stability. In addition to detecting potholes, the system accurately estimates pothole perimeter and depth by combining segmentation outputs with depth maps. The primary objective is to overcome the limitations of 2D vision-based methods by enabling accurate physical characterization of potholes while maintaining low computational complexity and real-time performance.

CONCLUSION: We conclude that their work effectively demonstrates the importance of geometry-aware segmentation and depth integration for reliable pothole detection and measurement. The use of DSConv and attention mechanisms improves boundary accuracy for irregular road defects, while segmentation-based depth analysis enables meaningful physical interpretation of potholes. From this study, we infer that precise boundary extraction and depth-informed modeling are essential for advanced road surface understanding. These techniques can be adapted to our project by employing similar segmentation strategies and structural analysis while replacing RGB-D sensors with monocular depth estimation, thereby supporting the development of a real-time wireframe pothole detection system focused on geometric representation and autonomous navigation support.

2.2 LITERATURE SURVEY – II

TITLE: Robust Video-Based Pothole Detection and Area Estimation for Intelligent Vehicles with Depth Map and Kalman Smoothing

AUTHORS: Dehao Wang, Haohang Zhu, Yiwen Xu, And Kaiqi Liu

YEAR: 2025

DESCRIPTION: This paper presents a vision-based pothole detection and classification system designed for autonomous driving applications. The authors use a deep convolutional neural network as a feature extractor and combine it with machine learning classifiers optimized using meta-heuristic algorithms. To address dataset imbalance, oversampling techniques are applied, improving classification robustness. The system focuses on accurate identification of potholes from road images under varying conditions and achieves high classification performance, demonstrating the effectiveness of deep feature extraction combined with optimized learning strategies.

CONCLUSION: We conclude that their work demonstrates the reliability of deep learning–based visual features for pothole detection in real-world scenarios. However, the approach is limited to two-dimensional classification and does not analyze the physical structure of potholes. The feature extraction and optimization strategies from this paper can be adopted in our project and extended with depth estimation and geometric modeling to enable structural understanding and wire-frame representation of road surface anomalies.

2.3 LITERATURE SURVEY – III

TITLE: Mobile3DRecon: Real-time Monocular 3D Reconstruction on a Mobile Phone

AUTHORS: Xingbin Yang, Liyang Zhou, Hanqing Jiang, Zhongliang Tang, Yuanbo Wang, Hujun Bao, Guofeng Zhang

YEAR: 2020

DESCRIPTION: This paper proposes a real-time 3D scene reconstruction framework using multiple RGB-D cameras to generate stable triangle meshes for dynamic environments. The authors integrate moving least squares (MLS) surface reconstruction with GPU-accelerated mesh generation to smooth noisy depth data and ensure temporal stability. The system efficiently produces consistent geometric representations suitable for real-time applications such as robotics and augmented reality.

CONCLUSION: We conclude that their work establishes an effective methodology for real-time geometric modeling and mesh generation. Although the system relies on RGB-D sensors, the concepts of surface smoothing, structural reconstruction, and real-time mesh generation are directly relevant to our project. These techniques inspire our wire-frame modeling stage, which adapts similar geometric principles using monocular depth estimation instead of dedicated depth sensors.

2.4 LITERATURE SURVEY – IV

TITLE: Pothole and Plain Road Classification Using Adaptive Mutation Dipper Throated Optimization and Transfer Learning for Self-Driving Cars.

AUTHORS: Amel Ali Alhussan, Doaa Sami Khafaga, El-Sayed M. El-Kenawy,
Abdelhameed Ibrahim, Marwa Metwally Eid, Abdelaziz A. Abdelhamid

YEAR: 2022

DESCRIPTION: This paper focuses on detecting road surface defects using deep learning–based computer vision techniques. The proposed approach leverages convolutional neural networks to learn robust texture and shape features from road images, improving detection performance under challenging conditions such as shadows, varying illumination, and surface noise. The study highlights the superiority of learned features over traditional handcrafted image processing methods.

CONCLUSION: We conclude that this work highlights the importance of robust feature learning for reliable road surface perception. While the system effectively detects defects, it does not provide depth or geometric interpretation of potholes. The insights from this paper support our use of deep learning for perception and motivate extending detection toward depth-aware and structural representations in our real-time wire-frame pothole detection system.

2.5 LITERATURE SURVEY – V

TITLE: Real-time scene reconstruction and triangle mesh generation using multiple RGB-D cameras

AUTHORS: Siim Meerits, Vincent Nozick, Hideo Saito

YEAR: 2017

DESCRIPTION: This paper presents an automated pothole detection approach using machine learning and computer vision techniques. The authors extract visual features from road images and apply classification models to identify potholes, emphasizing scalability and suitability for intelligent transportation systems. The proposed method demonstrates improved accuracy compared to traditional feature-based approaches.

CONCLUSION: We conclude that their study confirms the effectiveness of vision-based pothole detection for large-scale deployment. However, the lack of geometric and depth analysis limits its applicability to autonomous navigation. Our project builds upon this limitation by incorporating depth estimation and wire-frame modeling, enabling precise structural understanding of potholes for real-time decision-making in autonomous vehicles.

2.6 LITERATURE SURVEY COMPARISON:

| Sl. No | Title | Authors | Year | Technology Used | Conclusion |
| --- | --- | --- | --- | --- | --- |
|  | An Enhanced YOLOv8 Model for Real-Time and Accurate Pothole Detection and Measurement. | Mustafa Yurdakul, Şakir Taşdemir | 2025 | YOLOv8n-seg, DSConv, SimAM, GELU activation. | I conclude that integrating DSConv and SimAM into a YOLOv8 seg yields more accurate pothole masks and reliable physical measurements, which I can apply to my project to get precise segmentation masks and direct depth/perimeter estimates. |
|  | Robust Video-Based Pothole Detection and Area Estimation for Intelligent Vehicles with Depth Map and Kalman Smoothing | Dehao Wang, Haohang Zhu, Yiwen Xu, And Kaiqi Liu | 2025 | ACSH-YOLOv8, BoT-SORT, EMC, DepthAnything, MBTP, CDKF. | I conclude that integrating enhanced object detection, monocular depth estimation, pixel-level area computation, and Kalman-based smoothing can significantly improve the precision and reliability of pothole detection. |
|  | Mobile3DRecon: Real-time Monocular 3D Reconstruction on a Mobile Phone | Xingbin Yang, Liyang Zhou, Hanqing Jiang, Zhongliang Tang, Yuanbo Wang, Hujun Bao, Guofeng Zhang | 2020 | Monocular camera, 6DoF SLAM, SGM, DNN, TSDF voxel fusion, Mesh generation algorithm on mobile CPUs. | I conclude that their mobile-focused, CPU-based incremental mesh generation offers techniques and system design ideas directly applicable to resource-constrained 3D reconstruction tasks like pothole mapping or detection. |
|  | Pothole and Plain Road Classification Using Adaptive Mutation Dipper Throated Optimization and Transfer Learning for Self-Driving Cars. | Amel Ali Alhussan, Doaa Sami Khafaga, El-Sayed M. El-Kenawy, Abdelhameed Ibrahim, Marwa Metwally Eid, Abdelaziz A. Abdelhamid | 2022 | ResNet-50, Binary Advanced Mutation, Dipper Throated Optimization, SMOTE, Random Forest, AMDTO optimizer. | I conclude that applying meta-heuristic optimization for feature selection and classifier tuning together with an improved SMOTE balancing step can dramatically boost pothole vs plain-road classification accuracy. |
|  | Real-time scene reconstruction and triangle mesh generation using multiple RGB-D cameras | Siim Meerits, Vincent Nozick, Hideo Saito | 2017 | Point cloud smoothing, Mesh zippering, Direct triangle mesh generation, Mesh merging. | I conclude that their GPU-parallel mesh approach, gives insight into achieving fast, reliable surface modeling—enhancing the efficiency of mesh processing in pothole detection systems. |

Table 2.6: Literature Survey Comparison