Pothole Detection Prototype Guidebook

1. Purpose

This repository is a Python-based pothole detection prototype for an RC car workflow. It combines:

- live video capture from a webcam, video file, or ESP32-CAM stream

- pothole detection using either a heuristic detector or YOLO

- monocular depth estimation

- geometric approximation of pothole distance, width, length, and depth

- a small telemetry dashboard

- optional pseudo-label capture for online learning

- optional serial output to a motor-control microcontroller

This document is both:

- a developer guide for starting and understanding the project

- a practical manual outline for an end-user/operator document

2. Current Codebase Summary

The main runtime entry point is app.py, which forwards into src/pothole_detection/app.py.

Core subsystems:

- vision.capture: opens the video source

- models.detector: heuristic or YOLO pothole detection

- models.depth: proxy, MiDaS, Depth Anything, or hybrid depth estimation

- vision.wireframe: depth-based wireframe overlay and severity scoring inputs

- vision.geometry: approximates real-world distance and pothole dimensions

- control.policy: converts severity into CRUISE, SLOW, or AVOID

- control.serial_bridge: sends commands to a serial-connected controller

- dashboard.server: exposes telemetry over HTTP

- learning.online: stores high-confidence detections as pseudo-labeled samples

3. Prerequisites

Software

- Python 3.10 or newer

- pip

- OpenCV-compatible camera drivers if using a webcam

- Arduino IDE if you want to flash the ESP32 motor controller sketch

Optional ML/runtime dependencies

The project package only declares base dependencies:

- numpy

- opencv-python

Extra features require manual installation:

- pyserial for serial communication

- torch and torchvision for MiDaS

- ultralytics for YOLO training and YOLO inference

- transformers and pillow for Depth Anything

Hardware

Depending on the mode you want:

- laptop webcam, or

- USB camera, or

- ESP32-CAM streaming over Wi-Fi

For vehicle actuation:

- a motor-control ESP32 or Arduino-compatible board

- USB serial connection to the laptop

- motor driver / Adafruit Motor Shield hardware as expected by the current .ino sketch

4. Project Structure

app.py

pyproject.toml

datasets/

docs/

esp32/

scripts/

src/pothole_detection/

Important files:

- README.md

- docs/architecture.md

- src/pothole_detection/config/default.toml

- src/pothole_detection/models/detector.py

- src/pothole_detection/models/depth.py

- src/pothole_detection/dashboard/server.py

- esp32/esp32_rc_car/esp32_rc_car.ino

5. How to Start the Project

Step 1: Create and activate a virtual environment

Windows PowerShell:

python -m venv .venv

.venv\Scripts\Activate.ps1

Step 2: Install the package

pip install -e .

Step 3: Install optional dependencies based on your use case

For dashboard + heuristic detection only:

pip install pyserial

For YOLO and depth models:

pip install ultralytics pyserial torch torchvision transformers pillow

Step 4: Initialize the dataset folders

python scripts\init_dataset.py

This creates the YOLO-style dataset layout under datasets/pothole_yolo.

Step 5: Start the runtime

For the default heuristic pipeline using your local webcam:

python app.py --source 0

For a saved video:

python app.py --source path\to\video.mp4

For an ESP32-CAM stream:

python app.py --source http://192.168.4.1:81/stream

6. Supported Runtime Modes

A. Heuristic pothole detection

Default configuration:

- detection.mode = "heuristic"

- depth.mode = "proxy"

This mode is easiest to start because it does not require YOLO weights.

Run:

python app.py --source 0

B. YOLO-based pothole detection

Before using this mode you need:

- a trained .pt model

- ultralytics installed

- config changed so detection.mode = "yolo"

Run:

python app.py --source 0 --model-path runs\detect\train\weights\best.pt

Important: if detection.mode stays as "heuristic", passing --model-path alone does not activate YOLO.

C. Telemetry-only mode

This disables the OpenCV display window while keeping telemetry output active.

python app.py --source 0 --telemetry-only

D. Detection-disabled mode

This keeps the app alive, sends cruise commands, and updates telemetry without running pothole inference.

python app.py --source 0 --disable-detection --telemetry-only

E. Output recording

To save the annotated result video:

python app.py --source 0 --save-output artifacts\run.mp4

7. Command-Line Options

Supported options from the runtime:

- --source: camera index, video file path, or stream URL

- --config: custom TOML config path

- --model-path: YOLO weights path

- --serial-port: serial port for motor controller

- --baudrate: serial baudrate, default 115200

- --save-output: output annotated video file

- --telemetry-only: disable preview window

- --disable-detection: skip pothole inference

Example:

python app.py --source 0 --config src\pothole_detection\config\default.toml --serial-port COM5 --baudrate 115200

8. How the Project Functions Internally

The runtime loop in src/pothole_detection/app.py works like this:

1. load configuration from TOML

2. initialize dataset folders

3. open video source

4. build detector and depth estimator

5. start dashboard server

6. read each frame

7. run detection and depth estimation if enabled

8. compute geometry and severity

9. choose a vehicle command

10. write telemetry JSON

11. optionally write dashboard preview image

12. optionally send serial command to the controller

13. show the annotated frame unless telemetry-only mode is active

Severity mapping:

- low score: none or minor

- medium score: moderate

- high score: severe

Vehicle action mapping:

- none or minor -> CRUISE

- moderate -> SLOW

- severe -> AVOID

9. Dashboard

If dashboard support is enabled in config, the app starts an HTTP server at:

http://127.0.0.1:8765

The dashboard shows:

- detection status

- commanded vehicle speed

- pothole distance

- severity

- estimated width, length, and depth

- online-learning sample count

- FPS and detection count

Runtime state is written to:

- artifacts/dashboard/state.json

Optional frame snapshot is written to:

- artifacts/dashboard/frame.jpg

10. Configuration

Main configuration file:

- src/pothole_detection/config/default.toml

Key sections:

- [video]: display and frame size

- [camera]: field of view, mount height, tilt, depth scaling

- [runtime]: detection enabled and max detections

- [detection]: heuristic or yolo, thresholds, classes

- [depth]: proxy, midas, depthanything, or hybrid

- [severity]: score thresholds

- [control]: cruise/slow/avoid speeds and send interval

- [dataset]: dataset root and image extension

- [online_learning]: pseudo-label capture and retraining controls

- [dashboard]: dashboard host, port, preview behavior, telemetry file paths

Recommended first-run settings:

- keep detection.mode = "heuristic"

- keep depth.mode = "proxy"

- keep dashboard.enabled = true

- keep online_learning.auto_retrain = false

11. Training Workflow

The repository supports YOLO-format training data in:

datasets/pothole_yolo/

images/train

images/val

images/test

images/pseudo

labels/train

labels/val

labels/test

labels/pseudo

YOLO label format:

class_id x_center y_center width height

Typical training command:

yolo task=detect mode=train model=yolov8n.pt data=datasets\pothole_yolo\data.yaml epochs=50 imgsz=640

After training, update the config to detection.mode = "yolo" and run with --model-path.

12. Online Learning / Pseudo Labels

If online_learning.enabled = true and capture_enabled = true, the app periodically stores high-confidence detections into:

- datasets/pothole_yolo/images/pseudo

- datasets/pothole_yolo/labels/pseudo

This is a pseudo-label collection mechanism, not proof of correctness. Review those samples before using them for retraining.

Automatic retraining exists in code, but it is disabled by default. That is the safer configuration.

13. ESP32 / RC Car Integration Notes

The repository contains an Arduino sketch at:

- esp32/esp32_rc_car/esp32_rc_car.ino

Important mismatch in the current implementation

The Python app and the Arduino sketch are not yet protocol-compatible.

Python serial behavior:

- default baudrate is 115200

- sends lines like:

- CRUISE speed=55

- SLOW speed=30

- AVOID dir=right speed=24

Arduino sketch behavior:

- uses Serial.begin(9600)

- accepts:

- F

- B

- L

- R

- S

- STOP

Result:

- telemetry and perception can run

- serial output can print in mock mode if --serial-port is omitted

- full RC-car actuation will not work correctly until the command protocol is aligned

Recommended fix direction

Choose one of these:

1. change Python to emit F/L/R/S/STOP and use 9600

2. change the Arduino sketch to parse CRUISE/SLOW/AVOID at 115200

Until that is done, treat the serial-control portion as prototype/incomplete.

14. Known Limitations

- Default detection is heuristic, not a trained pothole detector.

- Distance/width/length/depth values are approximate and depend heavily on camera calibration.

- depth.mode = "proxy" is a synthetic depth approximation, not real metric depth.

- YOLO mode is only active when both config and --model-path are set correctly.

- Auto-retraining can reinforce bad pseudo labels if used carelessly.

- Current motor-control serial protocol is inconsistent with the shipped Arduino sketch.

15. Troubleshooting

App says dependency is missing

Install project and optional dependencies:

pip install -e .

pip install ultralytics pyserial torch torchvision transformers pillow

Camera cannot be opened

- verify camera index

- close other applications using the camera

- test a video file first

- verify the ESP32-CAM stream URL is reachable

Dashboard does not update

- confirm dashboard.enabled = true

- open http://127.0.0.1:8765

- check whether artifacts/dashboard/state.json is being created

YOLO does not run

- install ultralytics

- ensure detection.mode = "yolo"

- pass a valid --model-path

Serial commands are not moving the car

- verify COM port

- verify baudrate

- verify Python and Arduino are using the same command protocol

16. Recommended Developer Workflow

For a clean demo path:

1. run with webcam and heuristic mode first

2. verify dashboard output

3. verify pseudo-label capture folders are created

4. train or obtain YOLO weights

5. switch to YOLO mode

6. only then integrate serial hardware control

7. align the Python-to-ESP32 serial protocol before vehicle tests

17. What a User Manual Document Should Contain

If you want to produce a formal user manual for this project, include these sections:

Mandatory sections

- document title, version, author, and date

- project overview and objective

- target users

- hardware requirements

- software requirements

- installation steps

- startup procedure

- operating modes

- configuration instructions

- dashboard usage

- dataset/training instructions if the audience includes developers

- safety notes for RC-car operation

- troubleshooting

- maintenance/update process

- known limitations

- contact or support details

Strongly recommended sections

- system architecture diagram

- hardware connection diagram

- screenshots of the dashboard

- sample commands

- expected outputs

- serial communication notes

- FAQ

- glossary of terms

18. Suggested User Manual Template

Use this outline for a final .docx, .pdf, or report document:

1. Introduction

2. System Overview

3. Hardware Components

4. Software Installation

5. Folder Structure

6. Configuration

7. How to Start the Application

8. How to Run with Webcam

9. How to Run with ESP32-CAM

10. How to Use the Dashboard

11. How Detection Decisions Work

12. How to Train or Replace the YOLO Model

13. How to Collect Pseudo Labels

14. How to Connect the RC Car Controller

15. Troubleshooting

16. Known Issues and Limitations

17. Safety Instructions

18. Appendix

19. Suggested Short Operator Instructions

For a non-developer operator, the shortest usable runbook is:

1. activate the virtual environment

2. start the app with python app.py --source 0

3. open http://127.0.0.1:8765

4. watch detection status and severity

5. press q in the OpenCV window to stop

20. Final Assessment

This codebase is a workable prototype for:

- pothole visualization

- telemetry demonstration

- experimental geometry estimation

- future RC-car automation

It is not yet a complete end-to-end autonomous RC-car system because the current serial-control protocol between Python and the Arduino/ESP32 sketch is inconsistent.